FOOD SLICER

Policy:     	To prevent food borne illness, food service equipment shall be

cleaned according to standard procedures.

Policy Specifications:

| Procedure | Cleaning Materials |
| --- | --- |
| Frequency: After each use and weekly | Use “cut proof glove” when cleaning the Food Slicer. |
| 1.Turn machine OFF. Release safety lock if necessary. Unplug. |  |
| 2. Set blade control to zero. |  |
| 3. Remove meat carriage by turning knob at bottom of carriage. | . |
| 4. Remove back blade guard by loosening knob and the guard. |  |
| 5. Remove top blade by loosening center knob of blade. |  |
| 6. Take parts to pot sink and wash thoroughly. | Detergent solution. |
| 7. Rinse. Sanitize | Hot water. Sanitizing Solution. |
| 8. Air dry parts on clean surfaces. |  |
| 9. Wash blade and outside surface of machine plus electrical cord. Check hidden areas and corners. CAUTION: Do not allow water to enter electric unit or plug. | Detergent solution. Clean cloth. |
| 10. Rinse. Sanitize. | Hot water and cloth. Sanitizing solution. |
| 11. Air dry. |  |
| 12. Re-assemble unit. a. Immediately replace front blade guard after cleaning shell and tightening knob. b. Replace back blade guard and tighten knob. c. Replace meat carriage and tighten knob. |  |
| 13. Oil screw threads as directed in manual. |  |
| 14. Leave blade control to zero. |  |
| 15. Return to storage area that has been thoroughly cleaned. |  |

Note: Parts will be inspected prior to usage.

Cleaning solution:

Use the detergent and sanitizing solution set up for manual washing in the 3 compartment sink.

Sanitizing Solution:

Chlorine: 	  50 ppm-100 ppm

Quaternary:	 150 ppm-400 ppm