GRIDDLE

Policy:     	To prevent food borne illness, food service equipment shall be

Cleaned according to standard procedures.

Policy Specifications:

| Procedure | Cleaning Materials |
| --- | --- |
| Frequency: After each use and weekly |  |
| 1. Tum OFF and allow to cool. |  |
| 2. Scrape off food particles. Do not Scratch surfaces. | Scraper. |
| 3. Absorb excess grease. | Cloth or paper towels. |
| 4. Polish. | Soft griddle stone, screen, and materials as Needed. |
| 5. Remove grill stone crumbs. |  |
| 6. Lightly oil griddle. | Fresh cooking oil. |
| 7. Wash sides and back plate | Detergent solution/degreaser. |
| 8. Rinse and sanitize. | Hot water and sanitizing solution |
| 9. Apply rust prevention if made of cast , iron. |  |
| 10. Wash drain. | Detergent solution/degreaser. |
| 11. Empty drip pan. |  |
| 12. Wash, rinse and sanitize. | Detergent solution, hot water and sanitizer. |
| I 13. Air dry. |  |
| 14. Return to position. |  |

Cleaning solution:

Use the detergent and sanitizing solution set up for manual washing in the 3 compartment sink.

Sanitizing Solution:

Chlorine: 	  50 ppm-100 ppm

Quaternary:	 150 ppm-400 ppm